| Time | Location/Instructors | Module | Training Topics |
| --- | --- | --- | --- |
| 6:00 AM to 8:00 AM | Masonry Outdoor Lab/ «INSTRUCTOR(S)» | Hands-On Training Objective- Trainees will divided into 4 small work groups and assigned to one of the following stations to complete the assigned tasks/activities: - Mixing Mortar - Stocking material - Saw cutting - Scaffold | Group A : Mortar Mixing Station- Trainees will complete the following tasks: - Mixer Inspection & Setup - Mix a batch of mortar - Empty batch into wheelbarrow - Transfer to mortar boards - Tempering mortar - Clean up |
|  |  |  | Group B: Stocking Station- Trainees will practice stocking the following 30’ wall sections: - 8x8x16 gray w/ control joint |
|  |  |  | Group C: Saw Cutting Station (Table Saw & Quickie Saw)- Trainees will complete the following tasks: - Inspection & startup of saw - Table Saw cut- half blocks, open-ends - Quickie saw- center web, electrical box - Clean up of station |
|  |  |  | Group D: :Scaffold Station- Trainees will build the following scaffold section: - 4ft frames, 2 frames high, 28ft long w/ guard rail w/toe board on base plates |
| 8:15 AM to 8:30 AM | Construction Classroom | Break |  |
| Time | Location/Instructors | Module | Training Topics |
| 8:30 AM to Noon Please be sure to include the following breaks: 10:45 AM to 11:15 AM- Lunch | Masonry Outdoor Lab/ «INSTRUCTOR(S)» | Hands-on Training- Stocking Focus Trainees will spend the time learning how to stock materials for various wall sections. | Stocking Station- Trainees will practice stocking the following 30’ wall sections: - 8x8x16 gray w/ control joint - spilt one side 8x8x16 w/ control joint & corner - 12x8x16 w/ control joint - 8x8x16 w/ bond beams (2) w/control joint - 8x8x16 door w/ control joint - interlocking fence panel w/control joint - 4x8x16 veneer/ control joints - split /score pattern wall w/ control joint |
| Noon to 2:45 PM | Masonry Outdoor Lab/ «INSTRUCTOR(S)» | Hands-On Training Objective- Trainees will divided into 4 small work groups and assigned to one of the following stations to complete the assigned tasks/activities: - Mixing Mortar - Stocking material - Saw cutting - Scaffold | Group B : Mortar Mixing Station- Trainees will complete the following tasks: - Mixer Inspection & Setup - Mix a batch of mortar - Empty batch into wheelbarrow - Transfer to mortar boards - Tempering mortar - Clean up |
|  |  |  | Group C: Stocking Station- Trainees will practice stocking the following 30’ wall sections: - 8x8x16 gray w/ control joint |
|  |  |  | Group D: Saw Cutting Station (Table Saw & Quickie Saw)- Trainees will complete the following tasks: - Inspection & startup of saw - Table Saw cut- half blocks, open-ends - Quickie saw- center web, electrical box - Clean up of station |
|  |  |  | Group A: Scaffold Station- Trainees will build the following scaffold section: - 4ft frames, 2 frames high, 28ft long w/ guard rail w/toe board on base plates |