| Time | Location/Instructors | Module | Training Topics |
| --- | --- | --- | --- |
| 6:00 AM to 7:00 AM | Masonry Outdoor Lab/ «INSTRUCTOR(S)» | Hands-On Training Objective- Trainees will divided into 3 small work groups and assigned to one of the following stations to complete the assigned tasks/activities: - Mixing Mortar - Stocking material - Saw cutting | Safety Overview |
|  |  |  | Group A: Mortar Mixing Station- Trainees will complete the following tasks: - Mixer Inspection & Setup - Mix a batch of mortar - Empty batch into wheelbarrow - Transfer to mortar boards - Tempering mortar - Clean up |
|  |  |  | Group B: Stocking Station- Trainees will practice stocking the following 30’ wall sections: - 8x8x16 gray w/ control joint |
|  |  |  | Group C: Saw Cutting Station (Table Saw & Quickie Saw)- Trainees will complete the following tasks: - Inspection & startup of saw - Table Saw cut- half blocks, open-ends - Quickie saw- center web, electrical box - Clean up of station |
| 7:00 AM to 9:00 AM | Construction Classroom/ «INSTRUCTOR(S)» | Scaffold Module Objective- To provide trainees with a basic understanding of the components of a scaffold system and the proper methods for erecting and dismantling scaffold. | Scaffold Awareness Course |
| 9:00 AM to 9:15 AM | Construction Classroom | Break |  |

| Date | Duration/Location/Instructors | Module | Training Topics |
| --- | --- | --- | --- |
| 9:15 AM to 1:15 PM Include the following break: 11:00 AM to 11:30 AM- Lunch | Masonry Outdoor Lab/ «INSTRUCTOR(S)» | Hands-on Training Objective- Trainees will have the opportunity to build various scaffold sections | Trainees will practice building the following scaffold sections: - 4ft frames, 2 frames high, 28ft long w/ guard rail w/toe board on base plates - 4ft frames, 2 frames high, 28ft long w/ guard rail w/toe board on screw jacks - 6ft frames, 2 frames high, 28ft long w/ guard rail w/ toe board on base plates w/ side brackets - 6ft frames, 2 frames high, 28ft long w/ guard rail w/toe board on screw jacks w/side brackets |
| 1:15 PM to 1:30 PM | Construction Classroom | Break |  |
| 1:30 PM to 2:30 PM | 1 Hour/ Masonry Outdoor Lab/ «INSTRUCTOR(S)» | Hands-On Training Objective- Trainees will divided into 3 small work groups and assigned to one of the following stations to complete the assigned tasks/activities: - Mixing Mortar - Stocking material - Saw cutting | Group B : Mortar Mixing Station- Trainees will complete the following tasks: - Mixer Inspection & Setup - Mix a batch of mortar - Empty batch into wheelbarrow - Transfer to mortar boards - Tempering mortar - Clean up |
|  |  |  | Group C: Stocking Station- Trainees will practice stocking the following 30’ wall sections: - 8x8x16 gray w/ control joint |
|  |  |  | Group A: Saw Cutting Station (Table Saw & Quickie Saw)- Trainees will complete the following tasks: - Inspection & startup of saw - Table Saw cut- half blocks, open-ends - Quickie saw- center web, electrical box - Clean up of station |